Our 825 Avvonflo™ pump controller is a pre-configured unit designed to control the speed of diaphragm water pumps used in pole fed window cleaning equipment.

The controller functions by producing a pulse width modulated 12VDC pump supply which is controlled by a rotary potentiometer. Varying the pulse width adjusts the average voltage which raises or lowers the speed of the pump.

The basic controller control system is based upon our 725 series controller which was designed for a leading uk manufacturer and is in use daily in over a thousand machines throughout the country and abroad. It has a proven track record for reliability.

The 825 Avvonflo™  is our more advanced model and includes a number of innovative features.

The unit can be used with pumps with or without pressure switches. The pressure switch can be connected directly to the controller if required.

Also included is a remote flat battery warning led and pump shut-off function. This warns the end user that the battery needs to be recharged and prevents excessive discharge of the battery.

The Avvonflo™ retains rotary control for infinite adjustment over the useable range. Button based controllers cannot match the fine adjustment rotary control provides!

The Avvonflo™ controller is built into an ABS housing with dimensions of 120mm Long x 65mm Deep x 40mm high and is finished in light grey.

Fitted to the controller housing is a single pole function switch which is used as a function switch. It allows users who preserve the current flow rate setting between jobs.

Data Sheet Number 825-01/11

825 AVVONFLO™ PUMP CONTROLLER.

Design & Operating Characteristics

Connection Detail:

WIRE COLOUR:	FUNCTION:

Red			Positive Supply

Black			Negative Supply

Orange			Motor Positive

Grey			Motor Negative

White (pair)		To Pressure Switch

Mechanical & Technical Specification.

Weight:		Varies according to housing

Height:		40mm

Width:	 	120mm

Depth:		65mm

Adjustment:	Approx 20-95%

Ratings:		9-15VDC +/-1%

IP Rating:	IP64

FEATURES:

- Pulse Width Modulated Output.

- Ultra Efficient Mosfet Output Stage.

- Digital Bar Graph set speed display.

- On Led indicator.

- Low and Flat Battery Warning Led’s.

- Output on flying leads.

- Automatic Flat Battery pump shut-off.

- On-board automotive type plug-in fuse.

- Reverse polarity protected (Via Fuse).

- Robust industrial & repairable design.

- Function Switch preserves set speed.

CBC Design.

12 Jubilee Place, Clevedon,

UK. BS21 5EA.

Tel 07768 206649

Fax 01275 878362

Supplier:

CBC Design. Controller & Battery Charger Fabrication Refurbishment & Repairs.

12 Jubilee Place, Clevedon, North Somerset. BS21 5EA. Tel 07768 206649.

http://www.cbcdesign.co.uk