Data Sheet Number DM-01-10

12 & 24 Volt DC Dimmer Switches

& Fan Controllers

Design & Operating Characteristics

Connection Detail

Terminal No	Function 814 series

+V		To Positive Supply/Bulb +

0V		To Negative Supply

L-		To Lamp Negative

Terminal No	Function 714 series

+V		To Positive Supply.

0V		To Negative Supply.

L+		To Bulb Positive.

L-		To Bulb Negative.

Mechanical & Technical Specification.

# WALL PLATE VERSIONS 60, 150 & 300W

Weight:			120 Gram's

Height:			84mm.

Width:	 		84mm.

Plate Colour:		White, Chrome or Brass

Depth:			8mm.

Mounting Depth:		26mm.

## OEM VERSION              60W      120W,   300W

Weight in grams:		30	30         WOA

Height (mm):		24	24         35

Width (mm):		53	53         46

Depth (mm):		65	65         75

IP Rating:		N/A

Lamp Flicker:		Below Measurable limit

Adjustment:		20-100% Brightness

Ratings:			60, 150 or 300 Watts

12V Model:		Operates at 9-15VDC

24V Model:		Operates at 18-30VDC

The CBC 60W (12 & 24V 814) 120W (12 & 24V) & 300W (24V Only) DC Dimmer switches are a solid state device that allows single or multiple filament lamps to be adjusted from a dim glow to bright light. The fan controllers allow the speed of DC fans and small motors or pumps to be adjusted over a similar range to the dimmer switches.

Incorporating switch mode technology, the assemblies are efficient and easy to install. The assemblies have an efficiency of over 98% and run cool in all operating modes unlike linear based dimmers that waste energy by converting unwanted power to heat.

The units function by switching the DC supply at a specific frequency that is enough to reduce lamp flicker to zero. The switching creates a pulse width modulated output where the off-on time is increased or decreased to adjust the average voltage level and therefore lamp brightness or fan speed.

The CBC Dimmer is presented in two versions. The first of these (pictured) is a standard wall switch plate type finished in white, Chrome or brass. Other colours are available upon request. The second is for OEM applications where a manufacturer has their own panel and wants to incorporate a dimmer circuit or controller within the design. The adjustment potentiometer is on flying leads for integration into the manufacturers control panel.

Each version has a number of features including:

Fully adjustable output from a dim glow to bright light!

Saves battery power in dimmed modes!

Increases lamp life by applying lamp power slowly!

Easy to install with just 4 connections!

Integral on-off switch and adjustment potentiometer!

Inexpensive and reliable industrial design!

Utilises industrial grade components for long life!

12V  dimmers also works with MR16 12V Led Arrays!

Fan controllers can control DC diaphragm pumps!

Manufactured in the UK using lead free components!

With two versions to choose from, the dimmer can be used in a wide range of industrial, camping, automotive or marine applications where the ability to fully control lighting levels or fan speeds is required.

MANUFACTURER: CBC Design.

12 Jubilee Place, Clevedon, North Somerset. BS21 5EA. Tel 044 7768 206649

http://www.cbcdesign.co.uk